DECABROMODIPHENYL ETHER

SUPPORTING INFORMATION

Draft prepared by the ad hoc working group on decabromodiphenyl ether under the POPs Review Committee of the Stockholm Convention

20 March2015

Table 1.Non-exhaustive overview of sectors/ industries where decaBDE is used, identified end uses and applications1

| Sector/ industry | Applications | End use |
| --- | --- | --- |
| Electric and electronics industry | Electric and electronic equipment | - housings and internal components of TVs - mobile phones and fax machines - audio and video equipment - remote controls - communications cables - capacitor films - building cables - wire and cables, e.g., heat shrinkable tubes - connectors in E&E equipment - circuit breakers - coils of bobbins - printing and photocopy machine components - toner - cartridges and connectors - scanner components |
| Maritime, aviation and aeroneutic | Ships, boats, airplanes, space shuttles, rockets | - electrical wiring and cables - electric and electronic equipment - navigation and telecommunications equipment - computers and computer devices - audio and video equipment - electrical connectors - appliances - housings and internal components of TVs - fax machines - remote controls - communications cables - capacitor films - cables - circuit breakers - printing and photocopy machine components - toner - cartridges and connectors - scanner components - air ducts for ventilation systems - electrical ducts and fittings - switches and connectors - components in fans, heating fans and hair dryers - adhesive tape - composite materials e.g. epoxy |
| Private and public transportation | Automobiles/mass transportation | - fabric (where Deca-BDE is encapsulated in backcoating of article) - rear deck - upholstery - headliner - automobile seats - head rest - sun visor - trim panel - carpets - reinforced plastics - instrument panel - interior trim - under the hood or dash - terminal /fuse block - higher amperage wire & cable jacketing (sparkplug wire) - electric and electronic equipment - battery case and battery tray - engine control - electrical connectors - components of radio disk, GPS and computer - systems - other: - cars; - upper guard plates – pillars - upper panel – pillar - lower fender of right and left pillar - inner guard plates – door - lower guard panel – pillars, - car visor - down dashboard - front and rear bumper body - ventilation grill - outside rearview mirror, belts - wire harness vehicle roof - harn of antenna - harn of defrost - harn of tail door - engine harn - PE wiring sleeve in electrical harnesses - EPDM and PP coatings in fuel systems - Polymer components and housings (ABS/PP) - Low density polyurethane foams - Aramid tapes - Polyethylene naphthalate flexible circuits - Shrink tubes - In fabrics (backcoated) in the rear deck, upholstery, headliner, sun visor, head rest or trim panels - In reinforced plastics in the instrument panel and interior trims - Under the hood or dash in terminals / fuse blocks and in higher amperage wires & cable jacketing - In EEE, namely in battery cases and trays, engine controls, electrical connectors and components of radio disk, GPS and computer systems. - The same general categories (plastic components, circuit boards, textiles, upholstery and small EEE components) |
| Textiles and furniture | Textiles and furniture | - upholstery textiles e.g. sofas, offices chairs, mattresses - filters for cookers - blinds - draperies - blackout curtains - automotive textiles - geotextiles, wall coverings - households/furniture appliances - PU flexible foam - tents and tarps - interliners - foam fillings - carpets - other |
| Buildings/construction |  | - pipes - lamp holders - stadium seats - reinforced plastics - switches and connectors - facing laminates for insulation panel - film for use under the roof and to protect building areas - electrical ducts and fittings - components in analytical equipment in industrial and - medical laboratories - air ducts for ventilation systems - pillars for telephone and communication cables - waterborne emulsions with adhesive properties used for wall or floor coverings - piping insulation - cladding panels - composites - adhesives and adhesive tapes |
| Domestic | Household | - lamp sockets - kitchen hoods - electrical kitchen appliances - curtains and hanging drapes - components of water heating device - components of electrical appliances such as - transformers and switches - components in fans, heating fans and hair dryers |

1 Sources; BSEF 2006b, RPA 2014, US EPA 2014, Annex E Bulgaria.

Table 2. Overview over plastic types where c-decaBDE is used and their applications1

| Plastic | Type | Typical applications | Comments |
| --- | --- | --- | --- |
| Polypropylene (PP) | Polyolefin | Injection moulded parts, stadium seating, shipping pallets, roofing membranes, cladding panels |  |
| Polyethylene (PE) | Polyolefin | PE/wood composites, power cables, conduits, electrical connectors and boxes, wire and cable insulation, heat shrinkable material | PE foam can be used as thermal or acoustic insulation |
| Ethylene Vinyl Acetate (EVA) | Polyolefin / Elastomer | Wire and cable insulation, extrusion, coatings |  |
| High Impact Polystyrene (HIPS) | Polystyrenic | Plastic parts, panels, keyboards, casings, TV | Their use in EEE is restricted |
| Acrylonitrile / Butadiene Styrene (ABS) | Polystyrenic | General appliance moulding, car bumpers |  |
| Polyphenylene Oxide / Polystyrene blends (PPO/PS) | Polystyrenic |  |  |
| Polyethylene Terephthalate (PET) | Polyester | Textile fibres, plastic parts, switches, sockets, Electrical appliances |  |
| Polybutylene Terephthalate (PBT) | Polyester | Circuit breakers, sockets and electrical connectors, textiles, switches |  |
| Polyamides (nylon) | Engineering thermoplastic | In injection moulding for transport apps: (e.g. wheel covers and handles, chair and seat-belt mechanisms, under hood applications). High T engineering application, textile fibres, coils, electrical components |  |
| Polycarbonates (PC) | Engineering thermoplastic | Mirror housings, lights for cars, bumpers, window housings for trains and aircraft, casings, panels, keyboards | Including blends PC/ABS |
| Polyimides (PI) | Engineering thermoplastic | Bearings in aircraft, seals, gaskets |  |
| Melamine | Thermoset | Textile finishing |  |
| Unsaturated Polyester Resins (UPR) | Thermoset | Articles for construction (modular building parts, roofing materials, porch canopies, decorative mouldings) Fibre reinforced plastics, automobile parts |  |
| Epoxy resins | Thermoset | Adhesives, electronics, construction, aerospace |  |
| Ethylene Propylene Diene Monomer (EPDM) | Elastomer | Car radiator hoses, roofing membranes, cable and wire insulation |  |
| Styrene Butadiene Rubber (SBR) | Elastomer | Latex, carpet reinforcements, interior redecoration. |  |
| Thermoplastic Polyurethanes (TPU) | Elastomer | Automotive, wire and cable applications, gaskets |  |
| Waterborne emulsions and coatings |  | Adhesive applications (e.g. wall coverings, furniture, flooring), protective coatings, saturation of fibrous materials (paper, textiles) | e.g. acrylic, PVC, Ethylene Vinyl Chloride emulsions |

1Sources: ECHA 2012c, Plasticseurope 2013, ICL Industrial Products 2012 as cited in RPA 2014.

Table 3.Overview of flame retardants used in plastics and other polymers applied in products included in the EU RoHS-directive1

1Source. The Swedish Chemicals Inspectorate (2005). Survey and technical assessment of alternatives to Decabromodiphenyl ether (decaBDE) in plastics. Report No 1/05 and references therein; Klason C. Kubat J (2001). Plaster materialval og materialdata. 4th Edition, Fire FL (1991). Combustibility of plastic. Van Nostrand Reinhold NY USA, p-173-179, www.jamplast.com

Figure 12. Textile consumption in the EU by fibre type in 100 tonnes. The consumption is presented as statistics for total textile articles (A), clothing articles (B) and household articles (C) (Beton et al. 2014)

Source: KemI 2014, report 6/ 2014

Table 4: Percentage of samples shown to contain POP-BDEs or BDE-209 in a Dutch study1.

1POP Stream, R13-16, IVAM, The Netherlands, 2013.

Table 5. Concentration range of POP-BDEs and BDE-209in 90 study samples investigated in a Dutch study. Concentration in µg/g.1

1:POP Stream, R13-16, IVAM, The Netherlands, 2013.

Table 6. Shortlist of chemical alternatives identified in US EPA 2014and their hazard characteristics

Table 7. Potential alternatives for c-decaBDE identified in ECHA (2014)1 including information on Cas no., structure and use in different applications (polymers (plastic), textiles, coatings/adhesives).

| decaBDE |  | CAS No | Structure | Primary use/ technical feasibility | Environmental and health properties | Loading | Price2 | Economic feasibility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bis-(pentabromophenyl) ether (c-decaBDE) |  | 1163-19-5 |  | Polymers, textiles, coatings, adhesives | POP/PBT/vPvB | 12 % | €4/kg | - |
| No | Potential alternative substance. | CAS No | Structure | Primary use |  |  |  |  |
| 1 | Triphenyl phosphate (TPP) | 115-86-6 |  | Polymers | TPP does not meet the PBT criteriabased on its intrinsic properties. However, it shows some aquatic toxicity . Potential concern as an ECD. | 8-30 % | €3.2-6/kg | Could be more expensive than c-decaBDE because of the higher loading of the substance needed, and potentially higher price. Literature describes TPP as affordable. |
| 2 | Magnesium hydroxide (MDH) | 1309-42-8 |  | Polymers, (textiles), coatings/adhesives | Not classified as a PBT, as it is not an organic substance. It does not seem to raise any significant concern based on other hazardous properties | 45-60 % | €1/kg | Requires high loadings compared to c-decaBDE but it is likely to have a price per tonne lower than c-decaBDE |
| 3 | Tris(1,3-dichloro-2-propyl) phosphate (TDCPP) | 13674-87-8 |  | Polymers, textiles | TDCPP is not a PBT; however, it is toxic to the aquatic environment (H411) (with higher water solubility) and a Carc Cat 2 (H351). Recent research raises concerns about its potential neurotoxicity and female fertility effects | 1-12.6 % | €1-3.2/kg | Require marginally higher loadings compared to c-decaBDE but it is likely to have a price per tonne lower than c-decaBDE. It does not require ATO. |
| 4 | Aluminium trihydroxide (ATH) | 21645-51-2; 8064-00-4 |  | Polymers, textiles, coatings/adhesives | Insufficient information to conclude whether ATH pose a risk to Human Health and the Environment | 36-60 % | - | ATH requires very high loadings compared to c-decaBDE and the information on price per tonne is contradicting from different sources. |
| 5 | Tetrabromobisphenol A bis (2,3-dibromopropyl ether) | 21850-44-2 |  | Polymers (textiles) | Not a recognised PBT. However, it may show high persistence and there are concerns over its bioaccumulation. It may break down to TBBPA in the environment but how this may occur and what its significance is, are uncertain. Some concerns have been raised about its carcinogenicity but no conclusive results appear to be available in the literature | 1-14.5% | €2/kg | May require similar loadings to c-decaBDE. Information on prices from literature, Alibaba.com and consultation is conflicting. |
| 6 | Ethylene bis(tetrabromophthalimide) (EBTBP) | 32588-76-4 |  | Polymers, textiles, coatings/adhesives | Very persistent, may bioaccumulate and could potentially undergo debromination in the environment. Its human health hazard profile does not appear to raise concerns | 12-13% | €6.4/kg | Requires similar loadings to c-decaBDE but it is likely to have a price per tonne higher than c-decaBDE. |
| 7 | 2,2'-oxybis[5,5-dimethyl-1,3,2-dioxaphosphorinane] 2,2'-disulphide | 4090-51-1 |  | Textiles | Persistent, but not a PBT (based on screening information) and its human health hazard profile does not appear to raise concerns. | 10-15% | >€4/kg | Requires the same or lower loadings than c-decaBDE. However it has a higher price and requires the use of more expensive raw material fibres. As a result, the price of the textile articles are considerably higher than with c-decaBDE. |
| 8 | Resorcinol bis(diphenylphosphate) (RDP) | 57583-54-7; 125997-21-9 |  | Polymers (textiles) | Some concern about its persistence and bioaccumulation, but is not recognised as a PBT. It may show chronic and acute aquatic toxicity. Its degradation products are not known with certainty. | 8-20% | <€3.5/kg | May require lower loadings than c-decaBDE and it appears to have lower price per kg than c-decaBDE. However, the changes to the resins (e.g. from HIPS to a blend) and any other changes to the process may have a notable impact on costs, which literature has described as “affordable” |
| 9 | Bisphenol A bis(diphenyl phosphate) (BDP/BAPP) | 5945-33-5; 181028-79-5 |  | Polymers (textiles) | Some concern about its persistence, but is not recognised as a PBT. A classification for chronic aquatic toxicity is made. Degradation of the substance by dephosphorylation may generate substances of unfavourable hazard profile (bisphenol A, phenol), but this has not been proven. | 10-16% | €2-4.5/kg | Appears to be used at loadings similar to c-decaBDE and its price per tonne is assumed to be comparable to c-decaBDE. However, its use may require a change to different resin and notable investment costs |
| 10 | Substituted amine phosphate mixture (P/N intumescent systems) | 66034-17-1 |  | Polymers (textiles, coatings/ adhesives) | Limited information is available; persistence concerns and acute toxicity has been highlighted as potential problematic areas | 14.4-16.8 % | >€4/kg | Higher loadings and price per kg combined with changes to operating costs |
| 11 | Red phosphorous. | 7723-14-0 |  | Polymers, textiles, coatings/adhesives | May be considered persistent in the environment but as an inorganic compound cannot be subject to the PBT criteria. Aquatic Chronic 3 classification. With the exception of irritant properties and some uncertainty over genotoxicity and hepatoxicity, no significant concerns for human health appear to exist. Its physico-chemical hazards and the risk of disproportionation products are reduced through encapsulation and stabilisation | 2-12% | €3.2/kg | Limited information available indicates lower loadings and lower price per tonne than c-decaBDE |
| 12 | Ethane-1,2-bis(pentabromophenyl)(EBP) | 84852-53-9 |  | Polymers, textiles, coatings/adhesives | Highly persistent. There have been analyses on its bioaccumulation and reductive degradation and industry asserts that EBP does not behave like c-c-decaBDE and has provided information in support of this. It has been detected in many species from different habitats and trophic levels. | 10.4-30% | €4.2-4.8/kg | Used at the same loading as c-decaBDE and with the same presence of ATO. Its price is estimated to be 5-20% higher than c-decaBDE’s. Its use might require some minor process changes (whichhave not been explained during the consultation in detail). |
| 13 | 1,3,5-triazine-2,4,6-triamine (melamine) | 108-78-1 |  | Textiles | This alternative does not meet the PBT criteria.Based on the data available and relevant for the types of exposure involved, melamine seem to raise no significant human health concern. | - | - | - |

1Source: ECHA (2014). Annex XV Restriction report. Proposal for a restriction – Bis(pentabromophenyl) ether. Version number 1.0. 1. August 2014

2 Price was recalculated based on information in ECHA (2014).

Table 8. Potential alternatives for c-decaBDE identified in Annex F submissions1.

| No | Potential alternative substance. | CAS No | Primary use | Environmental and health properties | Loading | Price | Economic feasibility | Access and availability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Paxymer ® | - | Polymers; The technology replaces all halogenated flame retardants in polyolefin plastics (PP, PE, EVA etc). It works both synergistically with current technologies such as phosphorous/nitrogen (P/N) systems and mineral hydrate systems and as a stand alone system formulated with P/N base and the synergistic functionality.It has excellent, proven performance in PP and PE. Development efforts within rubber (EPDM), TPE and EVA materials are ongoing with optimistic results. Already implemented in a number of applications for various plastic processes. Proven processes are within injection moulding, calendaring, blow moulding and extrusion. Applications of flame retardant polyolefin plastics where c-decaBDE has been widely used are found in building, transportation and EEE industries. | No components of the Paxymer flame retardant system are classified as toxic, persistent, carcinogenic or endocrine disruptant in any way. Without PBT, CMR or EDC characteristics. | Addition levels of the stand alone product to achieve UL94-V0 are on par with DecaBDE and antimonytrioxide (synergist used with DecaBDE) and range between 25-32%. For conduits, wood polymer composites and building products lower addition levels have shown sufficient fire performance. Dependin on the fire standard, addition levels of between 15-25%. The synergistic system (especially with mineral based systems) has recommended dosage levels of 2-8% depending on the base material, (additional flame retardant systems and base material). | <12$/kg | Costs for coverters to facilitate full substitution if DecaBDE is limited. New technologies are largely developed and on par technically and commercially – especially if the entire product life cycle cost is accounted for (including waste management, handling of scrap in production and worker safety concerns). Several producers and global OEMs have already moved to phaseout of the substance and many have implemented completely halogen-free technologies. | Yes. Readily available in large quantities on the market. |
| 2 | F-2100 (Brominated Epoxy Polymer) | - | Polymers; Additive p olymeric brominated epoxy flame retardant recommended for use in PBT and PC alloys. | Doesn’t have PBT properties. It is not expected to have adverse health or environmental impacts. | - | - | - | Yes. Produced regularly in ICL-IP production plant. |
| 3 | FR-245 (2,4,6-Tris(2,4,6-tribromophenoxy)-1,3,5-triazine. | - | Polymers; Additive flame retardant. The combination of aromatic bromine and cyanurate provides high FR efficiency and good thermal stability. A major use of FR-245 is in ABS and HIPS. FR-245 combines good UV, impact and flow properties. | Doesn’t have PBTproperties. It is not expected to have adverse health or environmental impacts. | - | - | - | Yes. Produced regularly at ICL-IP production plant. |
| 4 | FR-720 (Tetrabromobisphenol-A, bis (2,3-dibromopropyl ether)). | - | Polymers; Additive flame retardant containing both aromatic and aliphatic bromine. Suitable for polyolefin and styrenic resins and it is especially recommended for UL-94 class V-2. It is also applicable in class V-0 polypropylene. | Doesn’t have PBTproperties. It is not expected to have adverse health or environmental impacts. | Efficient as c-decaBDE | - | FR- 720 is cost efficient and allows good physical properties of the resin to be maintained. | Yes. Toll Produced regularly by ICL IP |
| 5 | FR-1410 (Decabromodiphenyl ethane) | - | Polymers and textiles; Additive flame retardant containing 82% aromatic bromine. Its high bromine content coupled with its exceptional thermal stability makes it the material of choice for a large variety of applications. Major application areas include HIPS, Low-Density Polyethylene, Polypropylene (Homopolymers and Copolymers), Elastomers, PBT (Polybutylene terephthalate), Polyamides, UPE (Unsaturated Polyester) and Epoxy. FR-1410 is an alternative for c-decaBDE in PO (Polyolefin) + Textile applications | Doesn’t have PBTproperties. It is not expected to have adverse health or environmental impacts. | Similar to c-decaBDE. | - | - | Yes. Produced regularly in our production plant. |
| 6 | Declorane Plus |  |  |  |  |  |  |  |

Table 9. Non-chemical flame retardant material

| Alternative technique | Description |
| --- | --- |
| Intumescent systems | The mechanism of FRs based on intumescent technologies is to cause the plastic, when heated, to swell (intumesce) into a thick, insulating char that protects the underlying material from burning, by providing a physical barrier to heat and mass transfer. For intumescent technologies, the Danish EPA (2006) suggests that solutions for polypropylene (PP) have been commercially available for many years, but they face both technical and economic viability challenges. The substances concerned often belong to nitrogen-containing or organophosphorous flame retardants (e.g. phosphorous/nitrogen compounds, ethylenediamino phosphate etc.) |
| Nanocomposites | Mesoporous silicate particles (MSP) are porous silica beads, which when compounded with polymers can form a physically cross-linked polymer-particle network. During combustion, the network provides a char barrier that reduces flame intensity while simultaneously improving the mechanical performance of the polymer. When used on their own, they will not typically result in achieving flame retardancy, but by replacing a portion of the flame retardant loading with about 2 to 8% by weight MSPs, flame retardancy may be reached (US EPA, 2014). According to the UK HSE (2012), research into the use of nanocomposites has focused on plastics like polymethyl-methacrylate (PMMA), polypropylene, polystyrene, and polyamides, and they require special processing. For the time being, PINFA (Phosphorus, Inorganics and Nitrogen Flame Retardants Association) does not consider nanocomposites are viable standalone flame retardants. |
| Expandable graphite | On exposure to fire, the graphite expands to over 100 times its original size producing a barrier effect. It has been used in thermoplastics and can be used in polyolefins in combination with another FR such as ammonium polyphosphate, magnesium hydroxide, chloroparaffins or red phosphorous (UK HSE, 2012). According to industry, expandable graphite is used in plastics, rubbers (elastomers), coatings, polymeric foams and also in textiles. However, use of synergists is necessary to achieve the required flame retardancy. In some cases, process condition as well as equipment should also be modified (NYACOL, 2014). |
| Smoke suppressants | In the event of fire these systems lead to the formation of glassy coatings or intumescent foams or dilution of the combustible material, which prevents further formation of pyrolysis products and hence smoke (KemI, 2005). Such systems are of particular relevance to transportation applications of decaBDE. Molybdic oxide is one such substance and common FRs used alongside it include aluminium hydroxide and magnesium hydroxide (KemI, 2005). |
| Polymer blends | Readily flammable polymers (e.g. HIPS or ABS) may be blended with less readily flammable polymers such as PC, PPO or polyphenylene sulphide (PPS).This enables lower flame retardant loadings to be used with limited impact on other technical properties (UK HSE, 2012). The replacement of decaBDE in EEE enclosures stipulated by the RoHS Directive provide examples of the implementation of such solutions. More expensive polymers such as PC, PPO and PPS (possibly in the presence of a fluorinated polymer (as synergist) so that halogen-free FR options also become possible) may also be used as substitutes to the combination of HIPS/decaBDE/ATO or ABS/decaBDE/ATO while achieving an acceptable processability and recycling properties (JRC, 2007). |

Table 10. Non-chemical flame retardant material

Table 11. Durably finished and inherently flame retarding fibers in common use

Adapted from LCSP 2005

Table 13. Examples of alternative chemical treatments for c-decaBDE in textiles

Table 14. Conclusion on the hazard profile of the shortlisted alternatives to c-decaBDE and of their technical and economic feasibility1

| No | Alternative substance | CAS No | Hazard profile | Technical feasibility | Economic feasibility |
| --- | --- | --- | --- | --- | --- |
| 1 | Triphenyl phosphate (TPP) | 115-86-6 | aq tox, neurotoxicity(?), ED(?) | Polymers only | Higher price, similar concentration |
| 2 | Magnesium hydroxide (MDH) | 1309-42-8 | P by default (metal) | Wide range, but inefficient | Lower price, higher concentration |
| 3 | Tris(1,3-dichloro-2-propyl) phosphate (TDCPP) | 13674-87-8 | Carc.cat.2, neurotoxicity(?), effects on female fertility(?), P,T | Textiles, limited range | Lower price, similar concentration |
| 4 | Aluminium trihydroxide (ATH) | 21645-51-2; 8064-00-4 | Insufficient information | Wide range, but inefficient | Lower price, higher concentration |
| 5 | Tetrabromobisphenol A bis (2,3-dibromopropyl ether) | 21850-44-2 | P, B, degradation products, CMR(?) | Mostly polymers | Lower price, similar or lower concentration |
| 6 | Ethylene bis(tetrabromophthalimide) (EBTBP) | 32588-76-4 | vP, B, debromination(?) | Drop-in | Higher price, same concentration |
| 7 | 2,2'-oxybis[5,5-dimethyl-1,3,2-dioxaphosphorinane] 2,2'-disulphide | 4090-51-1 | Insufficient information | Viscose fibres only | Significantly higher price similar or lower concentration, requires more expensive raw materials |
| 8 | Resorcinol bis(diphenylphosphate) (RDP) | 57583-54-7; 125997-21-9 | P/vP, B(?) | Polymeric blends | Lower price, similar concentration, require changes in the production process |
| 9 | Bisphenol A bis(diphenyl phosphate) (BDP/BAPP) | 5945-33-5; 181028-79-5 | P(?), degradation products | Polymeric blends | Lower price, similar concentration, require notable changes in the production process |
| 10 | Substituted amine phosphate mixture (P/N intumescent systems) | 66034-17-1 | Acute tox, P | Polymers | Higher price, higher concentration, require changes in the production process |
| 11 | Red phosphorous | 7723-14-0 | P by default (metal, hepatotoxicity(?), Aq tox 3 but test results would support Aq Chr 1 | Certain polymers, cotton-rich textiles | Lower price, lower concentration |
| 12 | Ethane-1,2-bis(pentabromophenyl) (EBP) | 84852-53-9 | Subject to Substance Evaluation under REACH based on suspected PBT/vPvB properties | Drop-in | Higher price, same concentration |
| 13 | 1,3,5-triazine-2,4,6-triamine (melamine) | 108-78-1 | No significant concerns | technically feasible* | economically feasible* |

1Source: RPA (2014) and, where denoted by *, based on confidential information received from industry.

Table 15. Flame retardants identified in a screening for chemicals in textiles1

1Source: KemI 2014, report 6/ 2014

Table 16.The most viable alternatives found in the ENFIRO project1 among 14 halogen-free flame retardants tested as alternatives for decaBDE, tetrabromobisphenol A and brominated styrens.

1Source: ENFIRO http://www.enfiro.eu/

Abbreviations: Dihydrooxaphosphosphaphenanthrene oxide (DOPO), aluminium hydroxide (ATH), melamine phosphate (MPP), aluminium diethylphosphinate (Alpi), zinc hydroxyl stannate (ZHS), zinc stannate (ZS), resorcinol bis (biphenyl phosphate) (RDP), bis phenol A bis (phenyl phosphate) (BDP).

References

BSEF 2006b [to be completed]

ECHA, European Chemicals Agency (2014). Annex XV restriction report proposal for a restriction. Substance name: Bis(pentabromophenyl) ether, IUPAC name: 1,1'-oxybis(pentabromobenzene), EC number: 214-604-9, CAS number: 1163-19-5. European Chemicals Agency, Helsinki, Finland prepared in collaboration with the Norwegian Environment Agency. Version number: 1.0.

ENFIRO (2013). Final Report Summary - ENFIRO (Life Cycle Assessment of Environment-compatible Flame Retardants ( Prototypical case study)). Available at: http://cordis.europa.eu/publication/rcn/15697_en.html.

IVM (2013) POP Stream, POP-BDE waste streams in the Netherlands: analysis and inventory (R13-16). Institute for Environmental Studies, The Netherlands.

KemI, Swedish Chemical Agency (2014). Chemicals in textiles – Risks to human health and the environment. Report from a government assignment. Report 6/14, pp 1-140.

KemI, Swedish Chemical Agency (2005).Survey and technical assessment of alternatives to Decabromodipenyl ether (decaBDE) in plastics. Authors: Posner S, Börås. Report No 1/05, pp 1-34.

LCSP, Lowell Center for Sustainable Production (2005) Decabromodiphenylether: An investigation of non-halogen substitutes in electronic enclosure and textile applications. University of Massachusetts Lowell.

RPA, Risk&Policy Analysts.Georgalas B, Sanchez A, Zarogiannis P (2014). Multiple Framework Contract with Re-opening of competition for Scientific Services for ECHA. Reference: ECHA/2011/01 Service Request SR 14: Support to an Annex XV Dossier on Bis-(pentabromophenyl) ether (DecaBDE).

U.S. EPA US Environmental Protection Agency (2014a). An alternatives assessment for the flameretardant decabromodiphenyl ether (decaBDE).http://www.epa.gov/oppt/dfe/pubs/projects/decaBDE/deca-report-complete.pdf.